Direct Port IO Under Windows NT & 95

Warning !

This DLL and driver achieve direct access to ports in your PC and will defeat hardware security.

This capability allows you full access to any IO port.  Indescriminate use of these capabilities may lead to disastrous results.  Hewlett-Packard will not take any responsibility for anything that results in a actions detrimental to your PC's health.

Tested Systems

The systems I have tested this on are Windows NT™ 3.51 and Windows95.

Calls In the PeekPoke DLL

The PeekPoke DLL includes the following functions:

long openDriver(char *driverName);         // For NT only.

long writeByte(long port, long dataByte);

long writeWord(long port, long dataWord);

long writeLongWord(long port, long dataLong);

long readByte(long port);

long readWord(long port);

long readLongWord(long port);

Using PeekPoke

Under Windows 95, all you have to do to run the DLL from VEE is copy the Peekpoke.dll library

and the Peekpoke.vh VEE header file to your runtime VEE directory -- note that you don't have to

specify a ".h" extension for the VEE header file; beware that you don't confuse it with the different "Peekpoke.h" file that is used to compile the DLL.

Once this is done, then you can use the VEE Import Library function to specify the PeekPoke.h

and Peekpoke.dll in your VEE program, and then perform Call Functions to the routines.  You

do not use the openDriver function -- that's for NT only, see below.

Installing The Driver Under NT

Under Windows NT, you must first install a driver named 'giveio.sys' that opens up direct port access.

There is a utility called 'loaddrv' that allows you to do this.  You must be logged into an account that runs with system administrator privileges to load a driver.   The following is a screen shot showing how to use the driver loader:

Type in the fully-qualified path to the driver file you want to load.  Click the 'Install' button.  You should see an 'Operation Successful' message.

Click the 'Start' button. You should see an 'Operation Successful' message.

When you want to disable access to IO ports, click the 'Stop' button.

When you want to remove the driver from your operating system, click the 'Remove' button.

Note that when you do an Import Library on the DLL, before you do any calls to it under NT, you

must first do a Call Function on "openDriver", using "giveio" as the driver name.

An Example

This is a VEE program to test the DLL by clicking the built-in speaker on your PC.