# Installing computerboards PCMCIA-card

advise. 	Take at least driver Version 2.10

- Installing drivers from Computerboards disk (2 disks)

- plug in PCMCIA-card into the slot

- Choose "loading driver from disk". While doing this remain the first disk in drive (the system loads the file .inf

- under settings : system control : System click twice on ComputerBords GPIB. Then twice on PCM-GPIB. Under Resources note the I/O range and the interrupt address. Switch off the automatic Settings.

- Call Cbconfig under programs in start menu. Go to the point to set the GPIB0-card. When you choose the type PCM-GPIB and the values appear you wrote down before you won. If not chose board type ISA-GPIB and type in the values manually. In adition to this you have to choose primary address. It is recommended to chose 14. Store the data’s and exit the program.

- Call  Cbtest under programs in start menu aufrufen. Now you must see all thee tests with ok, this includes the interrupt test. If not check the steps before.

- Copy the files gpib.dll, gpib_16.dll und gpib_32.dll from path GPIB... to the path C:\windows\system.

- Now is the point to connect cables and devices

- Call  the VEE I/O-configuration  programm now. Call "my configuration"  + Refresh-Button. You should see now the entry "HP-IB14" with a new device under this point.

- Pushing OK-Button will send a string to the connected device. The result will be the device description.