ESCORT MEMORY SYSTEMS

A DATALOGIC GROUP COMPANY

HMS827 Demonstration program.

Version 1.2A

# Table of Contents

Table of Contents	1

Introduction	2

User overview	2

0. Read Continually.	3

1. Block Read.	3

2. Block Write.	3

3. Protected Block Write.	4

4. Search Tag.	4

5. Read tag ID.	4

6. Clear Tag.	4

7. Fill Tag.	5

8. Check antenna.	5

9. Configure Non-contiguous R/W.	5

A. Non-contiguous Read. Configuration assumed.	6

B. Non-contiguous write. Configuration assumed.	6

C. Change baud rate.	6

D. Infinite retry Block Reads.	6

E. Copy from a tag.	7

F. Write copied tag.	7

G. View copied data.	7

As a Software example	7

What is on the demonstration disk?	8

# Introduction

The HMS827 is an advanced passive reader/writer antenna.    It contains commands and features that are unique to the HMS827 antenna controller.    As an example of how the commands are used, an HMS827 demonstration program, has been included.    It is a standard DOS application.    Communications port one (1) or two (2) (COM1 or COM2) must be used to interface with the HMS827, in RS232 mode.    This demonstration program is kept simple and can be run in a DOS shell or true DOS.    The intention is to show how the HMS RFID interface commands can be used.

An HMS827 can receive commands from an RS232 interface or a MUX32 buss.    The basic demo.exe program is written to interface with an HMS827 using an RS232 interface.    These commands are specified in the Operator’s Manual.    Looking at the C source code, a compile time flag, in comm.c, can be seen. PRINT_CMDS enables development debugging code that dumps out commands sent to the antenna and the response received back.    Because that information may be useful for a technical individual interfacing with an HMS827, a copy of demo.exe with the compile time flag of PRINT_CMDS set to TRUE is on the disk under the name of demo_dbg.exe.    This demonstration program is intended to aid in development work.    It has been kept simple and assumes that the user is used to looking at debug output.

The demonstration program is written in standard ANSI C. Development work of the HMS827 demonstration program was done in a DOS environment, with a Borland C++ Version 4.02, Copyright (c) 1994, Borland International.    None of the Borland C++ or windows extensions have been used. So, it should work with any standard DOS C compiler.    The MAKE utility used for development is MAKE Version 3.7 Copyright (c) 1987, 1994 Borland International.    Although, because the demonstration program consists of only four (4) files, a simple batch file (cc_demo.bat) can be used .    See the makefile for the compile command line options and comments.

# User overview

The executable version of the demonstration program can be used to see commands successfully executed.    The source code can be analyzed while execution is being observed.    The commands begin demonstrated are specified in the Operator’s Manual for the HMS827 Series Passive Reader/Writer.    That manual should be referenced for a full description of the below commands.

When the demonstration program is executed, it checks to see if a communications port is specified on the command line.    A one (1) or two (2) can be on the command line to specify COM1 or COM2.    If no command line argument is given to the demo.exe program, then COM1 is assumed.    The serial communications port is used at 9600 Baud rate, with eight (8) data bits, one (1) stop bit, and no parity.

C:\hms827\demo :=-> demo 2

Version V1.0A.0, using COM2 at 9600 baud rate.

Data display format:

0. Hexadecimal

1. Decimal

2. ASCII

3. Binary

X: Exit

Select data display format:

When demo.exe is executed, the first menu brought up is the display format selection menu.    The selection made here is for both data input and data to be displayed.    The format chosen at this menu is only valid for data.    Other numbers, like address ranges, are always in decimal. It is from the display format menu that the demonstration program can be exited or after a display format is chosen, the command menu is brought up.

It is from the Command Menu that HMS commands are initiated. Commands in the HMS827 demonstration program perform the following:

## 0. Read Continually.

This is to continually reads data from the HMS tag, for the specified duration of time.    An entire tag is continually read and the data is displayed on the screen.    First, the duration of continual read is requested. ENTER at this prompt will use the default of one minute of continual read.    A number no greater than 32,767 seconds can be entered.    The read begins at address zero (0) and continues until the end of tag.    Then, the read begins again at address zero (0).    The ENTER key can be used to pause and then resume this read.    As with other commands, the ESC key can be used to abort the command and return to the menu.

## 1. Block Read.

This is to perform Block Read    (05H) commands.    Enough Block Read commands are issued to get and display the tag range specified.      The starting address for the first Block Read and then the number of data bytes to be read is requested.    No more than one display line of data is read from a tag in a given Block Read command. This command can be used to observe a single Block Read command.

## 2. Block Write.

This is to perform Block Write (06H) commands.    Enough Block Write commands are issued to write to the tag the entered number of bytes.      First, the starting address is requested and then the data is entered.    If the chosen format ASCII, then a string is terminated with the ENTER key.      If the display format is decimal, hexadecimal, or binary, then the data is entered in that given format.    The address for the data being entered is displayed followed by a colon(:).    The number is then entered followed by the ENTER key.    When enough data has been entered respond to the request for the next data byte with the ENTER key preceded with no digits.      That will initiate the Block Write commands.

## 3. Protected Block Write.

This is to perform Protected Block Write (0BH) commands.    This command is like a Block Write command, but the address allowed by the HMS827 is dependent on how the antenna controller is configured.    Enough Protected Block Write commands are issued to write the entered number of bytes. First, the starting address is requested and then the data is entered.    If the chosen format ASCII, then a string is entered and terminated with the ENTER key.      If the display format is decimal, hexadecimal, or binary, then the data is entered in that given format.    The address for the data being entered is displayed followed by a colon(:).    The number is then entered followed by the ENTER key.      When enough data has been entered respond to the request for the next data byte with the ENTER key preceded with no digits.      That will initiate the Protected Block Write commands.    If the address range specified is not in a protected region of the HMS827, an error code will be returned.

## 4. Search Tag.

This performs a Search Tag (08H) command.    This is a very simple command.    By using, demo_dbg.exe one can see a valid response, if a tag is present, or a sequence of retries if no tag is found.    If an antennas does not respond, then the demonstration program will eventually (a few seconds) time-out and try again.    This is a simple example of an application might interface with the HMS827 passive reader/writer antenna.

## 5. Read tag ID.

This reads and displays the tag’s four (4) byte identification number.      This identification number is displayed in both decimal and hexadecimal format.    The    display format chosen at the Data display format menu is not used to display the tag’s identification number. This identification number cannot be changed, it is not part of the 736 data bytes in the tag.

## 6. Clear Tag.

This performs a virtual clear tag function.    A clear tag is performed with the Fill Tag command (04H).    After the user is asked to verify that a tag is to be cleared, a tag is searched for and then filled with zeros(0).    A Search Tag command (08H) is followed by a Fill Tag command (04H), if a tag is found.

## 7. Fill Tag.

This performs a Fill Tag command (04H), with the data value entered by the user.    The starting address, number of data bytes to be filled, and the data value are requested before the command is issued.    Like the Clear Tag function, a Tag Search command is first performed, and then after a tag is found the Fill Tag command (04H) is issued.

## 8. Check antenna.

This command is intended to check to see if an HMS antenna is attached and can detect a tag being present.    That is done by continually issuing the Search Tag command (08).      The time-out value used in this application is about a quarter (1/4) of a second.    Thus, if the antenna is attached and no tag is present, the HMS827 antenna will respond more quickly.    If the HMS827 does not respond, then it is assumed that an antenna is not attached.    If the HMS827 responds, then the type of response (command echo or error code) indicates if a tag is present or not.

## 9. Configure Non-contiguous R/W.

This performs a Configure Non-contiguous R/W Address command (03H).    This command must be executed before any Non-contiguous Read or Non-contiguous Write can take place.    If any other command is issued between this configuration command and a Non-contiguous Read or Non-contiguous Write then the Read or Write will fail.    The tag addresses for the non-contiguous Reads are requested first.    After the desired number of tag addresses for non-contiguous reading have been entered, The ENTER key is entered alone (with no preceding digits).    Then the addresses for the non-contiguous writing are entered.    When the desire number of addresses for non-contiguous writing have been entered ENTER alone (with no digits) is pressed.    That initiates the configure non-contiguous R/W command.    See the HMS827 Operator’s Manual for a full description of Non-contiguous I/O and the maximum number of addresses that are allowed.    An example might look as follows:

Input read addresses (ENTER when finished): 10

Input read addresses (ENTER when finished): 20

Input read addresses (ENTER when finished): 30

Input read addresses (ENTER when finished): 40

Input read addresses (ENTER when finished):

Input write addresses (ENTER when finished): 10

Input write addresses (ENTER when finished): 40

Input write addresses (ENTER when finished):

Setting up for non-contiguous I/O. Please wait...

Non-contiguous configuration is done.

Hit  ANY  key to continue ...

## A. Non-contiguous Read. Configuration assumed.

This performs a Non-contiguous Read command (01H).    This is to read data in non-contiguous addressed in the tag.    It is assumed that a configure non-contiguous R/W command has been issued, during this execution of the demonstration program.    The non-contiguous addresses setup in the configure non-contiguous R/W for reading are used.    See the HMS827 Operator’s Manual for a full description of Non-contiguous I/O.

## B. Non-contiguous write. Configuration assumed.

This performs a Non-contiguous Write command (02H).    This is to write data to non-contiguous addressed in the tag.    It is assumed that a configure non-contiguous R/W command has been issued. Data values are requested, for the non-contiguous write addresses.    After the desire number of data bytes (number specified in the configure non-contiguous R/W command) have been entered, use the ENTER key alone (with no digits) to initiate the command.    If an incorrect number of data bytes where entered, the HMS827 will respond with an error code. See the HMS827 Operator’s Manual for a full description of Non-contiguous I/O.

## C. Change baud rate.

This sends the Change RS232 Baud Rage command (0BH) to the HMS827 and updates the baud rate in the PC being used.    The new communications port speed in the PC is displayed.    If this command is successfully received by the HMS827, other commands should be able to be executed at the new baud rate.

After this command is selected, the possible baud rates are displayed.    After a baud rate is selected, the command is sent to the HMS827 and the PC’s baud rate is updated.

When the HMS827 is powered up or reset, it reverts back to the baud rate setting from the configuration menu.    This command does not make a permanently change in the HMS827 configuration.

## D. Infinite retry Block Reads.

This initiates a infinite Block Read mode command (0CH).    The HMS827 will then enter or exit read standby mode. First, the number of bytes for the infinite retry Block Read command to be trying to read is requested.    Because this initiates one (1) read command, the number of bytes must fit in one command.    Then, the starting address for the read command is entered.    This sends the Initiate/Cancel Infinite Block Read command to the HMS827.      Based on the number of bytes to be read, the HMS827 will begin or stop looking for a tag to perform a read from.    See the HMS827 Operator’s Manual for full descriptions of the Initiate/Cancel Infinite block Read.

## E. Copy from a tag.

This uses a sequence of Block Read commands to read a range of data in a tag.    The data values are then stored in the computer RAM.    The data values from the tag can be viewed or written to another tag.

## F. Write copied tag.

This command is only available if a Copy from a tag has successfully completed.    This uses a sequence of Block Write commands to write the data copied in the above command to a tag within range of the HMS827 antenna.

The same address range specified when copying tag data is used when the data is written to a new tag.    When this command completes, it returns to the main menu.    If it is desired to write the same data to another tag, enter E again.

Searching for a tag is displayed.    Then, when a tag is found the data begins to be written.    Data is sent to the reader/writer in blocks.    The highest address of the data that was successfully written to is displayed.    Data is considered to be written to the tag when the entire data block is written.    Thus, if a tag is removed from the antenna before all of the data bytes have been written, locations above the number displayed on the Antares may have been altered.

## G. View copied data.

This command is only available if a Copy from a tag has successfully completed.    This does not initiate any HMS827 command.    It is to view the data copied using the above command.

When viewing copied data one screen full is displayed on the screen at a time.    The enter key is used to view the next page of tag data that had been copied.    The ESC key can be used to abort this viewing.

# As a Software example

The source code for the HMS827 demonstration program is included on the disk.    It is intended as an example of how to interface with the HMS827.    It was written in a DOS environment, using Borland C++ Version 4.02, Copyright (c) 1994, Borland International.    A make file and batch file are enclosed. Either can be used to perform the compilation and linking of the demonstration program.

The HMS827 demonstration program can be built to produce four (4) different executables.    The behavior of the executable that is produced from the source code provided is controlled by two (2) compile time flags, TEST_MUX32 and PRINT_CMDS.    The other BOOLEAN defines, like JANUS, ANTARES, or PORT_PC are for other HMS products and should be left as they are in the source code provide. TEST_MUX32 and PRINT_CMDS are the two defines of interest.

PRINT_CMDS is located in comm.c.    It is used to produce debugging dump information about demo.exe and HMS827 interaction.    When this flag is set to TRUE, what is sent to and received from the HMS827 is displayed.    Demo_dbg.exe is on the disk provided and was built with PRINT_CMDS being TRUE.    Demo.exe was built with PRINT_CMDS set to FALSE.

TEST_MUX32 is in handheld.h.      Demo.exe and demo_dbg.exe where built with TEST_MUX32 set to FALSE.    If TEST_MUX32 is set to TRUE, then the demonstration program sends commands that are preceded by an 0xAA and MUX32 address. The HMS827 responses should also be preceded by an 0xAA and MUX32 address.      This is to send HMS commands to an MM80 that has HMS.HEX loaded.    The MM80 will then send the message over the MUX32 buss to an HMS827.      To use the demonstration program with TEST_MUX32 set to TRUE requires having an MM80 with HMS.HEX loaded.    The PC sends commands to the local device connections on an MM80.    The HMS.HEX application loaded into the MM80 then sends the commands out the buss connections, over the MUX32 buss, to an HMS827.    See HMS.C with the MM80 for more on this MUX32 and HMS device interface.

A copy of the demonstration program with TEST_MUX32 set to TRUE is on the disk under the name demo_mux.exe.    That program will only function with an MM80 that has HMS.HEX downloaded.    It has an additional option in the display format menu, to set the MUX32 address of the HMS827that will be interfaced with.

## What is on the demonstration disk?

The following executable programs have been included on the disk to help you in development:

Demo.exe - RS232 interface, with no debugging information being dumped.

Demo_dbg.exe - RS232 interface, with debugging information being dumped on the screen.

Demo_mux.exe - MUX32 (RS485) interface, with no debugging information being dumped.

Demo_mdb.exe - MUX32 (RS485) interface, with debugging information dumped on the screen.

EC - directory containing:
EC.exe - Executable program.    Terminal Emulation program for configuring the HMS827.
EC.txt - Description of the EC program.